CS 3101 Computer Organization

Homework 4

Type your answers in this Word document. Write your name in the header. Rename the file to YourLastname_hw4.docx and submit in the correct folder in BlazeView.

Show your solution steps to receive partial credit.

- A digital computer has a memory unit with 30 bits per word. The instruction set consists of 58 different operations. All instructions have an operation code part (opcode) and two address fields: one for a memory address and one for a register address. This particular system includes eight general-purpose, user-addressable registers. Registers may be loaded directly from memory, and memory may be updated directly from the registers.  Direct memory-to-memory data movement operations are not supported. Each instruction is stored in one word of memory.

  - How many bits are needed for the opcode?

6 bits are required. Since it has a 58-operation 2^6

  - How many bits are needed to specify the register?

The problem has 8 registers, so you have 2^3, which means you have 3 bits for the register number

  - How many bits are left for the memory address part of the instruction?

In total we have 30 bits, so we subtract the ones we have already used. 30-6-3-1= 20. The 1 is to specify whether register or memory.

  - What is the maximum allowable size for the memory?

The maximum size will be 2^20 which is exactly 1 Mb

  - What is the largest unsigned binary number that can be accommodated in one word of the memory?

It should be 0*FFFFF in hexadecimal, because 20 bits equals 0*FFFFF in hexadecimal.

- List the hexadecimal code for the following program (hand assemble it), and predict what will be stored in memory location 10B after the program is executed.

Hex Address 		Label 		Instruction

100 					Load A

101 					Add One

102 					Jump S1

103 			S2, 		Subt One

104 					Store C

105 					Halt

106 			S1, 		Subt B

107 					Jump S2

108			One,		HEX 0001

109 			A, 		HEX 0022

10A			B,		HEX 0014

10B 			C, 		HEX 0000

Hex Address 		Label 		Instruction		Value

100 					Load A		0022 (A is loaded from 109)

101 					Add 0001		0023 (one is added)

102 					Jump S1		(skips all steps jump to S1)

106 			S1, 		Subt B			0015 (0014 – 0023)

107 					Jump S2		(skips all steps jump to S2)

103 			S2,		Subt 0001		0014 (0001 – 0015)

104 			 		Store C		0014 (0014 is stored in C)

105 					Halt			program will stop

In 10B the given value will be C which is equal to A+1 - B-1

A = 109 B = 10A

- Write the assembly language equivalent of the following MARIE machine language instructions:

  - Opcode [0100]  Value [010111000010]

[0100] is binary for 4 and stands for Subt X.

[0101] is binary for 5

[1100] is C

[0010] is binary for 2

The assembly language would be: Subt 5C2.

  - Opcode [0001] Value [001110011010]

[0001] is binary for 1 and stands for Load X.

[0011] is binary for 3

[1001] is binary for 9

[1010] is A

The assembly language would be: Load 39A.

  - Opcode [1100] Value [000101101100]

[1100] stands for Jump

[0001] is binary for 1

[0110] is binary for 6

[1100] is C

The assembly language would be: Jump 16C.