CS 3101 Computer Organization

Homework 5

Type your answers in this Word document. Write your name in the header. Rename the file to YourLastname_hw5.docx and submit in the correct folder in BlazeView.

- Show how the decimal value 2019111310 would be stored by byte-addressable machines with 32-bit words, using little endian format and then big endian format. Assume the value starts at address 201916. Draw a diagram of memory and place the appropriate values in the correct (and labeled) memory locations.

Convert 2019111310 In 32-Bit representation:

Bin= 0000  0001  0011  0100  0001  0111  1000  1001

Hex= 0	       1         3       4        1        7        8        9

Hexadecimal value: 0 x 01341789

Big Endian:

First byte is stored first.

2019	2020	2021	2022

01	34	17	89

Little Endian:

Last byte is stored first.

2019	2020	2021	2022

89	17	34	01

- Suppose we have the instruction Load 100. Given memory and register R1 contain the decimal values below:

| Memory Address | Content |
| --- | --- |
| 100 | 500 |
| 300 | 700 |
| 500 | 900 |
| 700 | 100 |
| 900 | 300 |

| R1 | 200 |
| --- | --- |

Assuming R1 is implied in the indexed addressing mode, determine the actual value loaded into the accumulator and fill in the table below:

| Mode | Value Loaded into AC |
| --- | --- |
| Immediate | 100. (100 is loaded.) |
| Direct | 500. (Address 100 = 500) |
| Indirect | 900. (Address 100=500=900) |
| Indexed | 700. (100+200(R1)=300=700) |

- A certain computer has a memory of 1M words, and each word is 32 bits long. Each instruction is 32 bits long and is consisted of an opcode field, a register address field to specify one of 32 registers, and a memory address field.

  - How large must the register field be?

In the problem is says that we have 32 registers. That equals 2^5=32. Since the 2 has a power of 5, it means that for every register we need 5 bits. The answer is 5 bits

  - How large must the address field be?

The problem says that there are 1M words that can be written as 2^20=1,048,576, which is roughly a million.  To address one individual word we would need 20 bits. The answer is 20 bits

  - How many different opcodes can be supported by this format?

32 bits- 20 bits – 5 bits= 7 bits.  We have 7 bits left to use in this problem.

So to summarize 32-bits:

Opcode	address field		registers

7 Bits		20 Bits			5 Bits

128=2^7	1M words		32 registers

Opcodes

Show your reasoning to receive partial credit.

- (bonus question, 5 points) In a computer instruction format, the instruction length is 12 bits and the size of an address field is 4 bits. Is it possible to design a coding method to support 13 2-address instructions, 46 1-address instructions, and 30 0-address instructions using the specified format? Show your design or argument to justify your answer to get credit.