CS 3101 Computer Organization

Homework 6

Type your answers in this Word document. Write your name in the header. Rename the file to YourLastname_hw6.docx and submit in the correct folder in BlazeView.

Show your solution steps to receive partial credit.

- Suppose a computer using direct mapped cache has 232 words of main memory, and a cache of 1024 blocks, where each cache block contains 32 words.

  - How many blocks of main memory are there?

Each cache block contains 32 words. 25= 32. What you have to do to find main memory is divide the fully associative cache which is 232 by the amount of words which is 25. 232/25= 227

  - What is the format of a memory address as seen by the cache, i.e., what are the sizes of the tag, block, and offset fields?

32 bit addresses with 17 bits in the tag field, 10 in the block field and 5 in the word field.

32 bit addresses is evident by the amount of words of main memory in the direct mapped cache. 17 bits in the tag field is evident by 32-(10+5)=17. 10 bits in the block field is evident by the number of blocks which is 210. 5 bits in the world field is evident by the number of words that each block contains.

  - To which cache block will the memory reference 0003101A16 map?

0*0003101A16 0*000003101A16   [0011 0001 00][01 1010]  00 1100 0001

0*3A15

- Suppose a computer using fully associative cache has 224 words of main memory and a cache of 512 blocks, where each cache block contains 16 words.

  - How many blocks of main memory are there?

Each cache block contains 16 words. 24= 16. What you have to do to find main memory is divide the fully associative cache which is 224 by the amount of words which is 24. 224/24= 220

  - What is the format of a memory address as seen by the cache, i.e., what are the sizes of the tag and offset fields?

24 bit addresses with 20 bits in the tag field and 4 in the word field.

24 bit addresses is evident by the amount of words of main memory in the fully associative cache. 20 bits in the tag field is evident by the number of blocks in the main memory. 4 bits in the world field is evident by the number of words that each block contains.

  - To which cache block will the memory reference 17042416 map?

Since it is an associative cache, it can map anywhere.

- (bonus question, 5 points) Suppose a computer using set associative cache has 216 words of main memory and a cache of 128 blocks, and each cache block contains 8 words.

a. If this cache is 2-way set associative, what is the format of a memory address as seen by the cache, that is, what are the sizes of the tag, set, and offset fields?

b. If this cache is 4-way set associative, what is the format of a memory address as seen by the cache?

- Main memory says that we have 16 bits in an address. 128=27, so cache contains 27, but each set contains 2 blocks, so we have 2^7/2, which is 26. Our 16-bit address is divided into 7 bits for the tag field, 6 bits for the set field, and 3 bits for the word field.

  - World field 2^3=8

  - Set field evident by 26

  - Tag field is found by 16-(6+3) = 7

- The 128 blocks in cache must now be divided into sets with 4 blocks each, implying we have 32 sets. The tag field would now have 8 bits, the set field 6 bits, and the word field 3 bits

  - World field 2^3=8

  - Set field evident by 26/2=25

  - Tag field is found by 16-(5+3) = 8