CS 3101 Computer Organization

Homework 7

Type your answers in this Word document. Write your name in the header. Rename the file to YourLastname_hw7.docx and submit in the correct folder in BlazeView.

Show your solution steps to receive partial credit.

- Suppose the daytime processing load consists of 60% CPU activity and 40% disk activity. Your customers are complaining that the system is slow. After doing some research, you have learned that you can upgrade your disks for $8,000 to make them 2.5 times as fast as they are currently. You have also learned that you can upgrade your CPU to make it 1.5 times as fast as the current CPU for $5,000.

a. Which upgrade option would give a faster system? Disks would give a faster system. because it has a 131.58% improvement while CPU has a 125% improvement.

b. Which upgrade option would give the best return of investment for each percent of speed improvement? CPU would be the better option for this, because it is $20.80 less than an upgraded Disk.  You only gain 6.58% of improvement by spending another $20.80 on an upgraded disk.

Show your calculation to support your conclusion.

CPU: 60 % disk activity is f = 0.6

Upgraded CPU speed is k = 1.5x

CPU upgrade costs $5,000

Disk: 40 % disk activity is f = 0.4

Upgraded Disk speed is k = 2.5x

Disk upgrade costs $8,000

Using Amdahl’s law S= (1)/((1-f)+(f/k))

To calculate upgrade costs= Upgrade/S

CPU activity: (1)/((1-0.6)+(0.6/1.5)) = 125%

CPU upgrade: $5,000/125% = $40

Disk activity: (1)/((1-0.4)+(0.4/2.5)) = 131.58%

Disk upgrade: $8,000/131.58% = $60.80

- Suppose a disk drive has the following characteristics:

• 6 surfaces

• 1024 tracks per surface

• 256 sectors per track

• 512 bytes/sector

• Track-to-track seek time of 8 milliseconds

• Rotational speed of 5000 RPM.

- What is the total capacity of the drive?

Capacity = surfaces x track x sectors x bytes

Capacity = 6 x 1024 x 256 x 512

Capacity = 805,306,368 which is roughly 800mbs

- What is its average access time?

Average access time = Average seek time + average rotational delay

Seek time is 8 msec

Time taken for one full rotation:

There are 5000 rotations in 60 seconds

1 rotation is 60 seconds

So you can make the equation 5000 times the number of rotations = 60

5000 * X = 60 seconds

X = 60/7500= 0.012, which is 12 milliseconds.

Rotational delay: ½ x 12 msec = 6 msec

Average access time = 8msec + 6msec = 14msec