CS 3101 Computer Organization

Homework 8

Type your answers in this Word document. Write your name in the header. Rename the file to YourLastname_hw8.docx and submit in the correct folder in BlazeView.

Show your solution steps to receive partial credit.

- Briefly describe the four primary models of computation in Flynn's taxonomy and give an example of a high-level problem for which each of these models might be used.

- SISD (single instruction stream, single data stream): execute a single instruction on one data value at a time. Example: classic von Neumann machines.

- SIMD (single instruction stream, multiple data streams): execute the same instruction on multiple data values at once. Examples includes anything that involves a Graphics processing unit.  Some examples include array processors and vector processors. It can be used for weather forecasting, medical diagnosing, and image processing.

- MISD (multiple instruction streams, single data stream): executes multiple instructions on one data value simultaneously. One example is a Space Shuttle.

- MIMD (multiple instruction streams, multiple data streams): consists of multiprocessors and most current parallel systems. Can be used in computer-aided manufacturing, simulation, and modeling.

- Describe write-through and write-back cache modification as they are used in shared memory systems, and the advantages and disadvantages of both approaches.

Write-through: In this system the memory content is updated as soon as the cache contents are modified. This happens because it broadcasts the changed data to all the processors in the system.  This way, they all update their specific cache block.

Advantages of write-through: Data is updated at the same time in main memory and cache.  This method is more reliable because all resources carry consistent information.

Disadvantages of write-thought: The disadvantages of this system is that it’s slow.  The CPU must wait for all the write in processes to be complete.  If the write buffer is full then the CPU and each cache must wait for it until it’s empty.

Write-Back: In this system the main memory is updated only when all the data in any caches has been changed.  The main memory will not change until one of the caches has the flag set.  So, whether you are removing or modifying data in the cache the main memory will not change until it is flagged.

Advantages of write-back: This process is faster because it waits until the cache is changed and not when it is replaced. Final data is uploaded to the main memory, which does not get in the way of processes.  This method does not allow for white stalls to occur.

Disadvantages of write-back: If only one cache is update but there are other cache’s in the system that are like the one you changed, that cache will not be changed.  This causes inconsistencies in the data.